AERIS

AI-Powered Autonomous Disaster-Response UAV

Full Concept Explanation and Prototype Plan

Prepared for Faculty / Mentor Discussion

# 1. Executive Overview

AERIS is an AI-powered autonomous disaster-response UAV designed to assist rescue teams during floods, earthquakes, landslides and other hazardous situations. The central idea is to move beyond using a drone only as a remotely operated camera. AERIS combines autonomous navigation, onboard AI perception, obstacle awareness, GPS geo-tagging, incident prioritization, ground-station communication and supervised response missions into one closed-loop robotic system.

The UAV can be assigned a search area or a specific location. It autonomously follows a planned mission, observes the environment through its camera, detects people and other relevant events using an edge-AI object-detection pipeline, records the location of detections and communicates actionable information to the rescue team. After human verification, the operator can assign a response mission to a detected location. Where the aircraft's payload capability permits, it can deliver a small emergency kit and then return or continue its search.

# 2. Problem Being Addressed

Disasters can make roads, buildings, bridges and open areas unsafe or inaccessible. Rescue teams may have limited visibility of the affected region and must spend valuable time locating people and assessing where help is required. Sending personnel into unknown or hazardous areas for initial reconnaissance can also expose them to additional risk.

The problem is therefore not simply finding a person. The larger problem is obtaining useful situational awareness quickly, converting observations into location-based actionable information, and enabling a response mission without unnecessarily exposing rescuers to the same hazards.

# 3. Proposed Solution

AERIS addresses this problem through an autonomous UAV that acts as an AI-assisted first-response robot. Its overall workflow is:

Receive a predefined search area or target location.

Take off and navigate using GPS-based autonomous flight.

Observe the environment using an onboard camera.

Run edge-AI detection on the companion computer.

Identify survivors and selected observable events such as vehicles, debris or fire/smoke when supported by the trained model.

Attach GPS position, timestamp and confidence information to detections.

Assign a basic priority level and transmit the incident to the ground station.

Require human confirmation for low-confidence or consequential detections.

Execute a supervised response mission to a confirmed location.

Where payload capacity permits, release a small emergency kit.

Return to base or continue the assigned search mission.

# 4. Where the System Can Be Used

| Scenario | Potential Use |
| --- | --- |
| Floods | Survey flooded zones, identify people in exposed locations, and provide geo-tagged leads. |
| Earthquakes | Survey areas with damaged or inaccessible roads and identify people or visible incidents. |
| Landslides | Inspect hazardous terrain without initially sending responders into the affected area. |
| Cyclones / severe storms | Perform post-event reconnaissance and locate people or visible hazards. |
| Urban emergencies | Rapidly inspect selected areas where access is restricted or unsafe. |
| Remote / hard-to-reach areas | Provide aerial reconnaissance where ground access is slow or difficult. |

# 5. How the Full System Works

| Stage | Operation |
| --- | --- |
| Mission Planning | A rescue operator defines a search area, waypoints or a target coordinate in the ground station. |
| Autonomous Flight | The flight controller manages stabilization, navigation, waypoint following and safety failsafes. |
| Perception | The camera captures the environment while the companion computer runs the AI detection pipeline. |
| Obstacle Awareness | The forward TFmini Plus measures distance to obstacles and provides basic forward obstacle awareness. |
| Detection | The AI model identifies supported objects such as people and other trained classes. |
| Geo-tagging | A detection is associated with the UAV's GPS position and time. |
| Prioritization | Confidence and mission context are used to create a basic priority level. |
| Communication | Detection and telemetry information are transmitted to the ground station. |
| Human Verification | Low-confidence detections and consequential response decisions remain subject to human approval. |
| Response | The operator can assign a confirmed target coordinate as a response waypoint; the UAV navigates there and can perform a supervised payload-release mission where physically supported. |
| Return / Continue | The UAV returns home or continues its assigned search mission. |

# 6. System Architecture

| Layer | Main Hardware / Software | Role |
| --- | --- | --- |
| Perception | Camera + YOLO-based AI | Detect people and selected objects/events. |
| Range Sensing | TFmini Plus | Forward obstacle-distance measurement. |
| Companion Computing | Raspberry Pi 5 + AI HAT+ | Run AI inference and high-level mission logic. |
| Flight Control | Pixhawk + ArduPilot | Stabilization, navigation, failsafes and actuator control. |
| Positioning | GPS | Navigation and geo-tagging. |
| Communication | SiK telemetry + RC/ELRS | Ground-station telemetry and manual safety control. |
| Response | Servo + small payload mount | Demonstration emergency-kit release. |

Core data flow: Camera → Raspberry Pi → AI HAT+ → Detection/Confidence → GPS association → Priority/Incident record → Ground Station → Human confirmation → Response waypoint → Pixhawk → UAV.

# 7. Full-System Capabilities / Features

| Capability | Status | Explanation |
| --- | --- | --- |
| Autonomous take-off / landing | Yes | Autonomous flight modes and configured safety procedures. |
| GPS waypoint navigation | Yes | Execute predefined routes and target coordinates. |
| Autonomous search | Yes | Search a defined area using a planned waypoint pattern. |
| AI survivor detection | Yes | YOLO-based visual detection on the companion computer. |
| Object/event detection | Yes, model-dependent | Vehicles, debris and selected visible events can be supported by trained classes. |
| Geo-tagging | Yes | Associate detections with UAV GPS position and timestamp. |
| Confidence scoring | Yes | Use model confidence to support triage and human verification. |
| Priority assessment | Basic | Mission rules can rank detections for responder attention. |
| Ground-station alerting | Yes | Transmit detection and telemetry information. |
| Forward obstacle detection | Yes | TFmini Plus. |
| Basic obstacle-aware navigation | Yes | Limited to the installed forward range-sensing configuration. |
| Search → detect → report | Yes | Core closed-loop reconnaissance workflow. |
| Search → detect → response | Yes | After human confirmation and within prototype constraints. |
| Emergency-kit delivery | Prototype-scale | Small payload demonstration only after safe flight validation. |
| Return-to-home | Yes | Configured failsafe/mission behavior. |
| Manual override | Yes | Human pilot retains emergency control. |
| Digital twin / simulation | Yes | Demonstrate scaling and test mission logic before physical deployment. |
| 5-kg emergency logistics | Future full-scale target | Not a capability of the small prototype. |

# 8. What Makes the Idea Different

The innovation is not the invention of a new drone airframe or a new object-detection algorithm. The innovation is the system-level integration of perception, localization, prioritization and response into one disaster-response workflow.

A conventional camera drone can provide video. AERIS is intended to convert observations into structured, geo-tagged and prioritized incident information and then connect that information to a supervised response mission.

SENSE → DETECT → GEO-TAG → PRIORITIZE → COMMUNICATE → RESPOND → RETURN / CONTINUE

The architecture is scalable. The small prototype validates the software and mission-control concept, while a future larger UAV can use higher-capacity propulsion, battery, sensing and payload hardware without changing the fundamental perception and mission-management layers.

# 9. PROTOTYPE — THE VERSION WE ARE BUILDING NOW

The prototype is a practical, lower-cost physical implementation of the core AERIS architecture. It is intended to prove that autonomous flight, AI perception, forward obstacle awareness, geo-tagged detection, communication and supervised response missions can operate together on a small UAV.

## 9.1 Prototype Airframe

A 450-mm F450 quadcopter is used as the physical platform. The prototype is deliberately smaller than the eventual heavy-lift disaster-response platform so that the available budget can be focused on reliable flight control, edge AI, sensing and integration.

## 9.2 Prototype Hardware

| Component | Proposed Model / Specification | Use |
| --- | --- | --- |
| F450 frame | 450-mm quadcopter frame | Main airframe. |
| Brushless motors | A2212 1000KV ×4 | Propulsion. |
| ESCs | 30A ×4 | Motor speed control. |
| Propellers | 1045 CW/CCW set | Generate lift. |
| Flight controller | Pixhawk 2.4.8 | Stabilization, navigation and failsafes. |
| Companion computer | Raspberry Pi 5 4GB | AI and high-level mission processing. |
| AI accelerator | Raspberry Pi AI HAT+ 13 TOPS | Edge inference acceleration. |
| Camera | Raspberry Pi Camera Module 3 | Visual perception. |
| Obstacle sensor | Benewake TFmini Plus | Forward distance measurement. |
| GPS | u-blox M8N-class GPS | Navigation and geo-tagging. |
| Telemetry | SiK telemetry pair | Ground-station communication. |
| RC receiver | ExpressLRS-class receiver | Manual safety override. |
| Battery | 3S 5200mAh LiPo | Main power; final selection validated against actual propulsion load. |
| Payload mechanism | Micro servo + lightweight mount | Small emergency-kit release demonstration. |

## 9.3 Prototype Software

| Software | Purpose |
| --- | --- |
| ArduPilot | Flight-control firmware and autonomous flight modes. |
| Mission Planner | Flight configuration, mission planning and telemetry. |
| MAVLink | Communication between companion computer, flight controller and ground system. |
| Python | AI integration, mission logic and data processing. |
| YOLO / Ultralytics + PyTorch | Object-detection pipeline. |
| OpenCV | Camera and image-processing functions. |
| Raspberry Pi OS / Linux | Companion-computer environment. |
| ROS 2 + Gazebo / ArduPilot SITL | Simulation and digital-twin development where used. |
| Blender / FreeCAD | 3D model and system visualization. |

## 9.4 How the Prototype Works

Operator defines a waypoint route or search area.

Pixhawk controls the quadcopter and executes the autonomous flight mission.

GPS provides position for navigation and incident geo-tagging.

The camera supplies visual data to the Raspberry Pi.

The Raspberry Pi + AI HAT+ runs the YOLO detection pipeline.

When a supported target is detected, the system records class, confidence, timestamp and approximate GPS position.

The ground station receives detection and telemetry information.

The TFmini Plus provides forward obstacle-distance information for basic obstacle-aware behavior.

A human operator verifies detections before consequential response actions.

The operator can assign a confirmed location as a response waypoint.

The UAV navigates to the location and can demonstrate a small payload release only after safe payload/thrust validation.

The UAV returns home or resumes the mission.

## 9.5 Prototype Capabilities

| Prototype Capability | Status |
| --- | --- |
| Autonomous GPS waypoint flight | Yes |
| Autonomous search pattern | Yes |
| AI-based person detection | Yes, after model integration/testing |
| AI-based selected object detection | Yes, depending on trained classes |
| Geo-tagging of detections | Yes |
| Ground-station telemetry | Yes |
| Forward obstacle detection | Yes, TFmini Plus |
| Basic obstacle-aware response | Yes, limited to forward sensing and tested logic |
| Human-in-the-loop confirmation | Yes |
| Targeted response waypoint | Yes |
| Small emergency-kit release | Planned demonstration; only after safe flight validation |
| Return-to-home/failsafe | Yes |
| Digital-twin/simulation workflow | Yes |
| 5-kg payload | No |
| 360° obstacle avoidance | No |
| 3D obstacle perception | No |

# 10. Prototype Limitations and Boundaries

The prototype is a proof-of-concept and should not be presented as a production search-and-rescue aircraft. The F450 platform is not intended to carry a 5-kg payload. The TFmini Plus provides forward distance sensing, so the prototype does not provide 360° or full 3D obstacle avoidance. Endurance, payload and wind performance must be established through actual flight testing.

The larger 5-kg-class disaster-logistics UAV is a future scaled architecture represented through simulation/digital-twin work, not a claim that the small prototype physically performs 5-kg delivery.

# 11. Example Mission

Consider a flood-affected locality where several roads are inaccessible. A rescue operator defines a search polygon. AERIS takes off and follows the planned route while the camera scans the area. The AI model detects a person near a flooded building and produces a high-confidence detection. The system associates the detection with the UAV's GPS position and sends the incident to the ground station. The operator verifies the lead and assigns the location as a response waypoint. The UAV navigates toward the location while using its forward range sensor for basic obstacle awareness. If the prototype has passed payload and flight-safety tests, it releases a small emergency kit and returns to continue the mission.

# 12. Final Summary for Faculty

AERIS is a low-cost, AI-assisted autonomous disaster-response UAV that connects aerial search with actionable response. Its core workflow is to autonomously navigate a disaster area, perceive the environment using onboard AI, identify and geo-tag important events, prioritize information for responders, and execute supervised response missions. The physical prototype focuses on proving this architecture using an F450 quadcopter, Pixhawk flight controller, Raspberry Pi 5, AI HAT+, camera, GPS, TFmini Plus and telemetry. The future full-scale system can extend the same architecture to a larger aircraft with greater endurance, advanced sensing and substantially higher payload capacity.