REVISION 1 — Corrected radio-system BOM, Pixhawk pricing, weight accounting and procurement budget

FINAL HACKATHON BUILD PLAN

AI-ASSISTED AUTONOMOUS
DISASTER RESPONSE UAV

Physical prototype + edge AI + autonomous missions + digital twin

Budget target: ₹50,000 + small contingency | Realistic procurement range: ~₹53,000–₹60,000

Price check: 13 August 2026

This document is deliberately conservative. It distinguishes what the physical prototype can prove from the full-scale target system.

# 1. Executive Decision — What We Are Actually Building

We are building ONE real, small autonomous quadcopter prototype, not three drones. The physical aircraft will demonstrate the core disaster-response mission architecture: autonomous navigation, systematic search, onboard AI perception, geo-tagging, incident reporting, basic obstacle sensing/avoidance, return-to-home, and—only if flight testing proves sufficient thrust margin—a small emergency-package release. The full 5-kg disaster-response UAV is a scaled target demonstrated through the digital twin/simulation, not a claim about the ₹50K aircraft.

Final project name: AI-Assisted Autonomous Disaster Response & Emergency Logistics UAV

Primary physical mission: Search and assess a defined disaster area autonomously.

Secondary physical mission: Navigate to a specified coordinate and, if safely supported, release a small demonstration payload.

Full-scale target: 5-kg-class emergency delivery platform shown through the digital twin.

## What the project is NOT

Not merely a survivor-detection drone.

Not a 5-kg heavy-lift UAV disguised as a ₹50K prototype.

Not a fully autonomous life-critical decision maker; payload release remains human-authorized.

Not a production-ready all-weather rescue aircraft.

Not a claim of globally novel drone hardware.

## Core mission chain

Mission assigned → autonomous takeoff → GPS/waypoint navigation → obstacle sensing → camera/AI perception → event detection → geo-tagging → priority/alert → continue mission or respond → return-to-home/failsafe.

# 2. Original Full Idea vs. ₹50K Hackathon Prototype

The original concept remains the same system. The budget changes the physical scale and the amount of capability we can honestly demonstrate. The project source defines the prototype as an AI-assisted autonomous disaster-response UAV combining autonomous flight, computer vision, survivor detection, geo-tagging, telemetry, disaster assessment and emergency payload delivery under human supervision.

| Capability | Original / full system | ₹50K-ish physical prototype | Digital twin |
| --- | --- | --- | --- |
| Autonomous takeoff/landing | Yes | Yes | Yes |
| GPS waypoint mission | Yes | Yes | Yes |
| Search pattern | Yes | Yes | Yes |
| AI perception | Yes | Yes (limited model/classes) | Yes |
| Survivor/person detection | Yes | Yes | Yes |
| Vehicle/building detection | Yes | Possible with one multi-class model | Yes |
| Fire/smoke/flood/debris assessment | Yes | Selected scenarios only | Yes |
| Geo-tagging | Yes | Yes | Yes |
| Priority/decision layer | Yes | Yes | Yes |
| Basic obstacle sensing | Yes | Yes | Yes |
| Advanced 3D obstacle avoidance | Yes | No | Yes |
| Emergency delivery | Yes | Small payload only if validated | Yes |
| 5-kg payload | Target | No | Yes / simulated |
| Long endurance | Target | No; ~8–10 min typical estimate | Yes / modelled |
| Human supervisory control | Yes | Yes | Yes |

## Source-supported prototype target

The project capability document specifies an initial working target of 1–2 kg payload, autonomous waypoint mission, person/object detection, live camera + AI detection, GPS-assisted navigation, basic LiDAR/ToF obstacle sensing, servo-controlled drop box, RC + telemetry, and a laptop ground station. It describes the architecture as scalable toward 5 kg.

# 3. Final Capability Set

| Capability | What it means | Physical prototype? |
| --- | --- | --- |
| Autonomous mission | Operator assigns a search area or target coordinate; autopilot executes the mission. | YES |
| Search-pattern flight | Grid/lawnmower-style waypoint mission over a defined area. | YES |
| Obstacle sensing | TFMini Plus provides forward distance information. | YES |
| Obstacle avoidance | Basic avoidance/stop/re-route logic; not 360° advanced avoidance. | YES, limited |
| AI perception | Edge object detection from live camera. | YES |
| Survivor detection | Person detection and incident alert. | YES |
| Event geo-tagging | Attach GPS/mission position to detections. | YES |
| Priority layer | Classify incidents as normal/high/critical based on configured rules. | YES |
| Live telemetry | Position, altitude, speed, battery, flight mode, mission status. | YES |
| Ground station | Map, mission planning, telemetry and AI alerts. | YES |
| Return-to-home | Automatic return under configured conditions. | YES |
| Failsafes | Battery, GPS, communication and mission safety logic. | YES |
| Delivery mission | Navigate to a given coordinate and release a small package if safe. | OPTIONAL/TEST |
| 5-kg emergency logistics | Full-scale target capability. | NO — digital twin |
| Disaster mapping | Aerial imagery and event overlays; advanced mapping is a later step. | BASIC |

## Example A — Autonomous search

Operator selects Search Area A. The drone takes off, flies the predefined grid, avoids a detected obstacle, runs the onboard object detector, finds a person, attaches the event to the drone's position, sends the alert to the ground station, continues searching, and returns when the mission or battery condition requires it.

## Example B — Point-to-point emergency delivery

Operator supplies Target B coordinates. The drone takes off, navigates to the coordinate, uses obstacle sensing, hovers at the delivery point, waits for human authorization, releases a small test package, confirms mission completion and returns home.

## Example C — Search → response → continue search

The drone searches Sector A, detects a survivor, geo-tags the location and alerts the rescue team. The operator authorizes a response mission. The drone navigates to the target, releases the demonstration kit if the payload system is validated, returns, and resumes the search mission.

# 4. System Architecture

The critical design rule is: the flight controller handles safety-critical flight; the companion computer handles AI and high-level mission logic.

| Layer | Responsibility / components |
| --- | --- |
| GROUND STATION | Mission Planner / QGroundControl; map; telemetry; live video; alerts; operator commands |
| COMMUNICATION | SiK telemetry for MAVLink; RC/ELRS for manual override; optional Wi-Fi for local development/video |
| FLIGHT CONTROLLER | Pixhawk 2.4.8 running ArduPilot Copter; stabilization, GPS navigation, waypoints, RTH, failsafes |
| COMPANION COMPUTER | Raspberry Pi 5 4GB; camera capture, MAVLink interface, geo-tagging, mission logic, logging |
| AI ACCELERATOR | Raspberry Pi AI HAT+ 13 TOPS; Hailo-8L; accelerated object detection |
| PERCEPTION | Raspberry Pi Camera Module 3 + TFMini Plus distance sensor |
| PROPULSION | F450 frame + 4× A2212 1000KV + 4×30A ESC + 1045 props + 3S LiPo |
| PAYLOAD | Lightweight servo release box; only used if flight testing confirms safe thrust margin |

## Data flow

Camera → AI HAT+ → object detections → Raspberry Pi → GPS/MAVLink association → priority logic → ground-station alert. In parallel, Pixhawk handles IMU/GPS/altitude/control/waypoints/RTH. TFMini Plus supplies distance information for obstacle logic.

# 5. Bill of Materials — Current Price Planning

Prices below are planning prices checked on 13 August 2026 from current Indian listings/search results. They are not guaranteed quotes: GST, shipping, stock, seller and import status can change the final bill. The realistic procurement range is approximately ₹54K–₹61K. The lean target is near ₹53K if lower-cost legitimate sources are available and the team already owns a charger/laptop/tools.

| # | Item | Recommended model/spec | Purpose | Planning price | Priority |
| --- | --- | --- | --- | --- | --- |
| 1 | F450 frame kit | F450 + 4× A2212 1000KV + 4×30A ESC + 1045 | Airframe + propulsion | ₹4,149 | Core |
| 2 | Flight controller | Pixhawk 2.4.8 + GPS/power combo | Stabilization, navigation, waypoints, RTH, failsafes | ₹9,000–₹14,300 | Core |
| 3 | Companion computer | Raspberry Pi 5 4GB | AI host, camera, MAVLink, mission logic | ₹10,510–₹11,200 | Core |
| 4 | AI accelerator | Raspberry Pi AI HAT+ 13 TOPS / Hailo-8L | Hardware AI inference | ₹6,854 | Strongly recommended |
| 5 | Camera | Raspberry Pi Camera Module 3 | Live vision / AI input | ₹2,389 | Core |
| 6 | Battery | 3S 11.1V 5200mAh LiPo, ~40C–80C | Main propulsion power | ₹2,561 | Core |
| 7 | RC transmitter | RadioMaster Pocket ELRS | Manual override / pilot safety | ₹6,899 | Core |
| 8 | Telemetry | SiK telemetry pair, 433/915 MHz as legal/available | MAVLink drone ↔ ground station | ₹3,940 lean target; higher for branded sets | Core |
| 9 | Obstacle sensor | Benewake TFMini Plus | Forward distance measurement | ₹3,266–₹4,650 | Core |
| 10 | Cooling | Raspberry Pi Active Cooler | Prevent Pi thermal throttling | ₹500–₹800 | Required |
| 11 | Storage | 64/128GB high-endurance microSD | OS, models, logs | ₹500–₹800 | Required |
| 12 | Charger | 3S balance LiPo charger | Safe battery charging | ₹1,200–₹1,800 | Required |
| 13 | Power/wiring | BEC/regulator, XT60, wires, connectors, mounts | Power distribution and integration | ₹1,000–₹1,500 | Required |
| 14 | Payload | Servo + lightweight drop box | Small demonstration delivery | ₹400–₹700 | Optional |
| 15 | Spares | 2–4 prop sets, fasteners, heat-shrink, cable ties | Field reliability | ₹800–₹1,500 | Required |
| 16 | RC receiver | RadioMaster RP1/RP2 ELRS Nano Receiver | Aircraft-side ELRS receiver | ₹800–₹1,500 | Core |
| 17 | Transmitter battery | 2× 18650 Li-ion cells | Power for RadioMaster Pocket transmitter | ₹300–₹500 | Core |

Lean BOM target: ~₹54,000 before price/shipping variation. Safer procurement budget: ~₹56,000–₹61,000.

If the budget ceiling is hard at ₹50,000, the first things to cut are the optional payload mechanism and nonessential accessories—not flight safety, battery quality, cooling, or the AI architecture.

# 6. Why These Components / What They Handle

| Component | Why selected | Main load handled | Main limitation |
| --- | --- | --- | --- |
| Pixhawk 2.4.8 | Low-cost ArduPilot-capable autopilot | Flight stabilization, navigation, failsafes | Old hardware; not ideal for advanced production UAV |
| Raspberry Pi 5 4GB | Enough general compute for companion tasks | Camera, MAVLink, mission logic, logging | CPU alone should not carry all AI inference |
| AI HAT+ 13 TOPS | Moves supported neural inference to Hailo NPU | Object detection / vision inference | 13 TOPS is moderate; use one optimized model |
| Camera Module 3 | Compact, supported Raspberry Pi camera | Visual perception | No thermal imaging; lighting matters |
| TFMini Plus | Small, low-power, 12m-class range sensor | Forward obstacle distance | Single-point range; not 3D/360° perception |
| A2212 1000KV + 1045 | Cheap, widely available prototype propulsion | Lift for light quad | Not heavy-lift; keep AUW low |
| 3S 5200mAh | Correct voltage class for A2212 1000KV setup | Energy storage | Only ~8–10 min expected under realistic loaded flight |
| RadioMaster Pocket ELRS | Reliable manual override at reasonable cost | Pilot safety / override | Requires compatible receiver and configuration |
| SiK telemetry | MAVLink-oriented ground link | Mission/telemetry | Range depends on version, antennas, RF conditions and local rules |

## Important correction: do NOT use 6S with this A2212 1000KV setup

The selected A2212 1000KV motor is normally used in a 2S–3S configuration with 1045-class propellers. Therefore the final prototype uses 3S. If we want 4S/6S for a longer/endurance or higher-performance design, we must redesign the motor/prop/ESC combination rather than simply putting a higher-voltage battery on the current motors.

# 7. Software / AI / Technology Stack

| Layer | Technology | Use |
| --- | --- | --- |
| Flight firmware | ArduPilot Copter | Autonomous flight, waypoint missions, stabilization, RTH, failsafes |
| Flight protocol | MAVLink | Drone ↔ companion computer ↔ ground station communication |
| Ground station | Mission Planner (primary) / QGroundControl (alternative) | Mission planning, telemetry, parameters, logs |
| OS | Raspberry Pi OS 64-bit | Companion computer operating system |
| Language | Python | AI orchestration, MAVLink integration, geo-tagging, mission logic |
| Computer vision | OpenCV + Picamera2/rpicam | Camera acquisition and preprocessing |
| AI model | Ultralytics YOLO small/edge model; compiled to Hailo HEF where supported | Person/object detection |
| AI runtime | HailoRT / Raspberry Pi AI HAT+ software stack | NPU inference |
| Model training | PyTorch/Ultralytics on development PC | Fine-tune selected classes if needed |
| Telemetry/data | MAVLink + pymavlink / MAVSDK-Python as appropriate | Position, status, commands |
| Mapping/dashboard | Python web UI or lightweight local dashboard | Map, alerts, mission status |
| Digital twin | Gazebo/ArduPilot SITL or another compatible simulator; Blender/CAD for visualization | Virtual full-scale system and mission testing |
| CAD | Fusion 360 / FreeCAD | Airframe/payload mounts and digital-twin geometry |
| Version control | Git + GitHub | Code/version tracking |
| Logging | ArduPilot logs + JSON/CSV/SQLite | Flight and AI event records |

## AI model recommendation

Use ONE optimized multi-class detector rather than multiple independent models. Start with a small YOLO model and classes that can actually be demonstrated: person, vehicle and a small number of selected disaster indicators. Do not promise reliable fire, flood, debris, building-damage and survivor classification simultaneously unless the dataset and field tests support it.

The Raspberry Pi AI HAT+ supports 13 or 26 TOPS variants and is designed for local neural inference; Ultralytics documents exporting supported YOLO models to Hailo HEF for edge deployment. This makes Pi 5 + AI HAT+ a much better architecture than running all neural inference on the Pi CPU.

# 8. Can the Main Components Handle Multiple Operations?

Yes, if the workload is divided correctly. No, if we try to run every AI function on the Raspberry Pi CPU at once. The system should be intentionally partitioned.

| Task | Processor | Concurrent? | Reason |
| --- | --- | --- | --- |
| Flight stabilization | Pixhawk | YES | Independent real-time flight loop |
| GPS/IMU/altitude fusion | Pixhawk | YES | Flight-controller responsibility |
| Waypoint/RTH/failsafes | Pixhawk | YES | Safety-critical autonomy stays off the Pi |
| Camera capture | Pi | YES | Lightweight pipeline |
| YOLO inference | AI HAT+ | YES | NPU offloads supported neural inference |
| GPS association | Pi | YES | Low computational load |
| Priority logic | Pi | YES | Rule-based/high-level logic is lightweight |
| Telemetry | Pi + SiK | YES | Low bandwidth compared with video |
| Video streaming | Pi | LIMITED | Keep resolution/bitrate reasonable |
| Logging | Pi + Pixhawk | YES | Use asynchronous/buffered logging |
| Advanced mapping | Pi/laptop | NOT during heavy AI flight | Do heavier post-processing on ground |
| Multiple neural networks | AI HAT+ | LIMITED | 13 TOPS is moderate; prefer one optimized multi-class model |

Recommended concurrency rule: one optimized AI detector + separate range sensor + Pixhawk flight control. Do not stack several heavy neural networks on the Pi CPU.

## Thermal management

Raspberry Pi 5 can thermal-throttle under sustained heavy load, so active cooling is not optional for this aircraft. The official Raspberry Pi documentation recommends active cooling for heavy continuous workloads. The AI HAT+ is designed to stack with the Pi 5 and its active cooler.

# 9. Flight Time — Realistic Estimate

Do not use the theoretical battery capacity as the claimed flight time. A 3S 5200mAh battery is approximately 11.1 V × 5.2 Ah = 57.7 Wh nominal. If we conservatively use about 75% of that energy for flight safety, about 43.3 Wh is available. At a system draw of roughly 250–350 W, the simple energy estimate is about 7.4–10.4 minutes. Real wind, battery sag, propulsion efficiency, payload and aggressive maneuvering can reduce this.

| Average electrical power | Idealized 75%-usable estimate | Interpretation |
| --- | --- | --- |
| 250 W | ~10.4 min | Very light/efficient hover conditions |
| 275 W | ~9.4 min | Reasonable target |
| 300 W | ~8.7 min | Good planning assumption |
| 325 W | ~8.0 min | Loaded/windy/less efficient |
| 350 W | ~7.4 min | Heavy or aggressive operation |

Design target: ~8–10 minutes of useful flight, not 15–20 minutes.

With a small payload and wind, plan around ~6–8 minutes of conservative mission time until flight tests prove otherwise.

## How to extend endurance

First reduce mass. Every gram removed improves hover efficiency and payload margin.

Use a larger-capacity 3S battery only after weighing it. A larger battery adds energy but also adds mass; the gain is not linear.

Do not parallel two 5200mAh packs on this small F450 unless the propulsion calculation is redone; the extra ~0.8 kg can erase the endurance benefit.

For a serious endurance upgrade, redesign the propulsion around a larger frame, lower-KV motors, larger efficient propellers and a higher-voltage pack. Do not simply change the battery voltage on the current A2212 motors.

Reduce hover/forward speed, avoid aggressive acceleration, and use efficient waypoint paths.

Run only the necessary AI model; keep video resolution/bitrate controlled.

Use mission-level battery reserve: e.g., return-to-home threshold rather than draining the pack.

For long endurance, the next hardware generation should be a purpose-built 4S/6S propulsion system rather than the F450/A2212 setup.

# 10. Weight / Thrust Engineering Check

A2212 1000KV + 1045 performance depends on propeller, battery, ESC, motor quality and test conditions. Using the commonly quoted ~0.8 kg maximum/test thrust per motor gives a theoretical total near 3.2 kg for four motors. That number is not a design target; real flight should preserve substantial thrust margin.

| Mass item | Planning mass |
| --- | --- |
| Frame + motors + ESC + props | ~0.55–0.60 kg |
| 3S 5200mAh battery | ~0.40–0.45 kg |
| Pixhawk + GPS + power | ~0.10–0.12 kg |
| Raspberry Pi 5 | ~0.05 kg |
| AI HAT+ + mounting | ~0.05–0.08 kg |
| Camera + TFMini + telemetry + receiver | ~0.05–0.10 kg |
| Wiring/mounts/BEC/spares | ~0.10–0.15 kg |
| Estimated all-up mass | ~1.30–1.55 kg |
| Small payload target | ~0.25–0.30 kg only if test-supported |

Receiver weight note: the RP1/RP2 aircraft receiver is approximately 1–2 g and is negligible in the aircraft weight budget. The RadioMaster Pocket transmitter and its 2×18650 batteries are handheld equipment and are NOT included in aircraft all-up weight.

The exact all-up weight must be measured after assembly. The drone should not be approved for autonomous payload flight until actual static thrust, hover current, battery voltage sag, motor/ESC temperature and flight behavior are measured.

# 11. Hackathon Eligibility & Rule Fit

Based on the supplied Robofest Gujarat 6.0 guidelines, the project is strongly aligned with the event. The guidelines explicitly include rescue/disaster management/public safety and autonomous aerial robotics among the focus areas, and list AI/ML, computer vision, autonomous navigation, sensor fusion, embedded systems, edge computing and digital twins as suitable technologies.

| Requirement / criterion | Our project | Assessment |
| --- | --- | --- |
| Eligible participant type | Full-time STEM undergraduate team | ALIGNED |
| Team format | 4 students + 1 faculty mentor from same institution | Must comply |
| Real-world challenge | Disaster response / public safety | DIRECT MATCH |
| AI-powered robotic solution | Edge AI + autonomous UAV | DIRECT MATCH |
| Functional prototype allowed | Real flying UAV | YES |
| Hybrid physical + virtual allowed | Physical UAV + digital twin | YES |
| AI / perception / autonomous operation | YOLO + mission logic + ArduPilot | YES |
| 5-minute final pitch/demo | Search + response + digital twin | YES |
| Pre-built components | Allowed if declared before event | YES |
| On-site assembly/integration | Need to perform required integration on-site | MUST COMPLY |
| Pre-trained models | Allowed | YES |
| Battery safety | Commercial LiPo + approved charger + safe charging | MUST COMPLY |
| Indoor flight | Only in approved testing sessions | MUST COMPLY |

Important: the guidelines say pre-built CAD, hardware components, subassemblies and program code may be brought in only if declared before the event, while instrumentation, motor/driver/sensor integration, final assembly, final code activation and operation of the final prototype must be completed on-site. This needs to be planned with the team before the event.

Caution: the score ranges below are rough directional estimates, not a real forecast. There is no jury scoring dataset available to justify precision at the individual-point level.

# 12. Realistic Score Assessment

This is an estimate, not a prediction. The actual jury score depends on competitors, demo quality and how well the final implementation matches the claimed capabilities.

| Judging category | Weight | Conservative target | Why |
| --- | --- | --- | --- |
| Problem relevance, innovation & impact | 20 | 16–18 | Disaster response is directly relevant; system integration is useful, but the individual technologies are not novel. |
| AI integration | 15 | 11–13 | Real edge AI is strong; score falls if detection is generic or unreliable. |
| System design & engineering | 15 | 11–13 | Good separation of Pixhawk/Pi/AI HAT; old Pixhawk and basic obstacle sensing limit ceiling. |
| Hardware prototype & performance | 25 | 17–21 | A genuinely autonomous flying demo can score well; crashes, weak flight time or unstable payload will hurt heavily. |
| Digital twin/simulation | 15 | 9–12 | Strong if it is a real mission simulation, weak if it is only a 3D animation. |
| Hardware + digital twin integration | 5 | 3–4 | Need to show same mission logic/data assumptions across physical and virtual systems. |
| Presentation/demo/docs | 5 | 4–5 | A crisp 5-minute mission demo can score strongly. |
| Likely total | 100 | ~71–86 | Execution-dependent. |

Best-case with reliable flight, convincing AI, real obstacle response, small delivery demo, strong digital twin and clean presentation: potentially mid/high-80s.

If the aircraft is unreliable or the demo becomes a laptop-only AI presentation: the score can fall into the 50s–60s.

# 13. Why It Could Win — and Why It Could Lose

## Why it could score highly

Direct match to rescue/disaster management and autonomous aerial robotics.

It demonstrates a real physical robot rather than only a software dashboard.

The AI is onboard/edge rather than dependent on cloud inference.

The mission is more meaningful than 'detect a person': search → understand → geo-tag → respond.

A physical prototype plus digital twin matches the hybrid direction encouraged by the hackathon.

The architecture cleanly separates safety-critical flight control from high-level AI.

The same system can support both search and point-to-point response missions.

The project has a clear scalability story from a small prototype to a 5-kg disaster-logistics platform.

## Why it might NOT win

Autonomous disaster drones are not a globally novel concept; judges may have seen similar ideas.

YOLO + Raspberry Pi + Pixhawk + GPS is a common robotics stack.

A basic F450 prototype does not look like a professional 5-kg rescue UAV.

8–10 minutes of flight time is short for a real disaster mission.

A single TFMini is not true 360° obstacle avoidance.

The ₹50K budget prevents thermal imaging, high-end LiDAR, long-range cellular links and heavy-lift propulsion.

Generic object detection can look like a technology demo rather than a disaster-specific AI system.

If the digital twin is just a visual animation, it will not justify a high simulation score.

If the drone crashes or autonomous flight is unreliable, the 25% hardware category becomes a major weakness.

If we claim 5 kg without physically proving it, credibility will be damaged.

## How to improve the competitive position

Build a disaster-specific decision layer, not only a detector.

Use one optimized multi-class model and report measured precision/recall/latency on the selected classes.

Record real flight logs: flight time, hover current, GPS accuracy, detection FPS, inference latency and mission completion rate.

Demonstrate an actual search mission rather than a manually flown camera demo.

Demonstrate basic obstacle response with a controlled, safe test.

If the payload is not safely supported, do not fake it—show the full delivery workflow in the digital twin.

Make the digital twin consume mission parameters and AI event data rather than merely showing a 3D drone.

Show measurable improvement over manual-only operation: time-to-locate, area searched, incidents logged and mission completion.

# 14. Recommended Build Timeline

| Phase | Duration | Deliverable |
| --- | --- | --- |
| 1. Procurement + bench setup | 2–3 days | All hardware verified; battery/charger safety checked |
| 2. Frame/propulsion assembly | 1–2 days | Motors/ESCs tested; correct motor direction and prop orientation |
| 3. Pixhawk + ArduPilot | 2–3 days | Stable manual/assisted flight, GPS, RTH, failsafes |
| 4. Raspberry Pi + AI HAT+ | 2–3 days | Camera + Hailo runtime + test YOLO model |
| 5. MAVLink integration | 2 days | Pi reads GPS/status and sends high-level commands |
| 6. Ground station | 1–2 days | Mission planning + telemetry + alerts |
| 7. Geo-tagging + decision layer | 2–3 days | Incident record + priority + map marker |
| 8. Obstacle sensor | 1–2 days | Distance sensing + safe response logic |
| 9. Payload mechanism | 1 day | Bench test; flight test only if safe |
| 10. Digital twin/SITL | 3–5 days | Full-scale mission scenario |
| 11. Integrated testing | 4–7 days | Repeated autonomous missions; logs and metrics |
| 12. Demo hardening | 2–3 days | Fallback scripts, spare props, batteries, clean 5-minute demo |

Total practical preparation: roughly 3–5 weeks of part-time work for a team that already knows the basic software stack. More time is safer; do not plan to integrate the entire aircraft for the first time during the 36-hour event.

# 15. Final 5-Minute Demo Plan

Problem: rescue teams need rapid intelligence and response in areas that are dangerous or inaccessible.

Mission assignment: operator selects a search sector.

Autonomous takeoff and waypoint search.

Obstacle sensing/response.

AI detects a person/target and produces a geo-tagged incident.

Ground station displays the event, confidence, coordinates and mission status.

Operator authorizes a response mission if the small payload mechanism has passed safety tests.

Drone navigates to the target, performs the small release demonstration if supported, and returns.

Digital twin shows the same mission scaled to the full 5-kg emergency-delivery concept.

Close with measured results and limitations—not marketing claims.

## Metrics to show on the final slide

| Metric | Target / measurement |
| --- | --- |
| Autonomous mission completion | ≥ 90% across repeated controlled trials before event |
| Useful flight time | ~8–10 min target; publish measured value |
| AI inference | Measure FPS/latency with final model |
| Detection quality | Report precision/recall on selected test set |
| GPS event tagging | Report typical coordinate error under test conditions |
| Obstacle response | Report successful controlled trials |
| Mission repeatability | At least 5–10 repeated autonomous runs before final demo |
| Payload | Only publish measured safe payload, not theoretical maximum |

# 16. Sources and Verification Notes

Hackathon source documents supplied in the conversation:

Disaster_Response_Drone_Capabilities.pdf — operational workflow, capability chain, prototype target and safety architecture.

Robofest Gujarat 6.0 guidelines text — eligibility, problem areas, permitted technologies, 36-hour rules, evaluation criteria and safety requirements.

## Current hardware / technical references checked 13 Aug 2026

| Reference | Use | URL |
| --- | --- | --- |
| Raspberry Pi 5 | Official product/documentation + Indian listing | https://www.raspberrypi.com/products/raspberry-pi-5/ |
| Raspberry Pi AI HAT+ | Official specs: 13/26 TOPS; Hailo NPU | https://www.raspberrypi.com/products/ai-hat/ |
| Raspberry Pi AI HAT documentation | Official deployment/camera integration | https://www.raspberrypi.com/documentation/accessories/ai-hat-plus.html |
| Raspberry Pi Active Cooler | Official cooling guidance | https://www.raspberrypi.com/products/active-cooler/ |
| ArduPilot companion computer | Official MAVLink companion architecture | https://ardupilot.org/dev/docs/companion-computers.html |
| ArduPilot Copter | Official autonomous mission/RTH documentation | https://ardupilot.org/copter/docs/copter-use-case-overview.html |
| Ultralytics Hailo | YOLO → Hailo HEF deployment documentation | https://docs.ultralytics.com/integrations/hailo |
| F450 kit | Current Indian listing used for planning | https://www.theengineerstore.in/products/f450-quadcopter-frame-kit-with-a2212-kv1000-brushless-motor-and-4-30a-esc-and-2-pair-1045-propeller |
| Pixhawk 2.4.8 | Current Indian listing used for planning | https://robu.in/product/pixhawk-px4-autopilot-pix-2-4-8-32-bit-flight-controller/ |
| TFMini Plus | Benewake specs / Indian listings | https://en.benewake.com/TFminiPlus/index.html |
| RadioMaster Pocket | Official product page | https://radiomasterrc.com/products/pocket-radio-controller-m2 |
| SiK Telemetry | Holybro official product page | https://holybro.com/products/sik-telemetry-radio-v3 |

## Price caveat

Retail prices are volatile. The BOM is a procurement planning estimate, not a quotation. Before purchase, verify stock, GST, shipping, seller authenticity, connector versions, battery dimensions, and the exact voltage/current specifications.

# 17. FINAL LOCKED SPECIFICATION

| Item | Final decision |
| --- | --- |
| Project | AI-Assisted Autonomous Disaster Response & Emergency Logistics UAV |
| Physical platform | F450-class quadcopter |
| Flight controller | Pixhawk 2.4.8 + ArduPilot |
| Companion computer | Raspberry Pi 5 4GB |
| AI accelerator | Raspberry Pi AI HAT+ 13 TOPS |
| Vision | Raspberry Pi Camera Module 3 |
| Obstacle sensing | Benewake TFMini Plus |
| Propulsion | 4× A2212 1000KV + 30A ESC + 1045 |
| Battery | 3S 5200mAh LiPo |
| Radio | RadioMaster Pocket (TX) + RP1/RP2 Nano Receiver (RX) + 2×18650 TX batteries |
| Telemetry | SiK MAVLink telemetry |
| AI | One optimized multi-class YOLO model; Hailo deployment where supported |
| Software | ArduPilot, MAVLink, Raspberry Pi OS, Python, OpenCV, Ultralytics, HailoRT, Mission Planner/QGroundControl, SITL/Gazebo |
| Physical mission | Autonomous search + geo-tagged AI detection + reporting + point-to-point response |
| Physical payload | Small demonstration payload only if flight tests prove safe |
| Digital twin | Full-scale disaster search + response + 5-kg payload mission |
| Expected flight time | ~8–10 min planning range; publish measured result |
| Procurement budget | ~₹54K lean / ~₹56–61K safer |
| 5-kg claim | Digital twin/target only; NOT claimed for physical prototype |
| Innovation claim | System-level disaster-response integration, not novel drone hardware |
| Competitive risk | Common hardware stack + short endurance + limited obstacle sensing + limited payload |
| Primary success condition | Reliable autonomous live demo with measured evidence |

Bottom line: This is a feasible hackathon prototype if the team treats it as an engineered system, not a shopping list. The aircraft must be flight-tested before adding AI/payload complexity. The AI must be optimized and accelerated. The digital twin must be functional, not decorative. And every capability shown to judges must be one the team can actually reproduce.